ByteSizedDemo on Nuvoton NuMaker-IIoT-NUC980

# Hardware (as provided by Nuvoton)

# Connector and Components

1 -> micro-USB connector: connect to USB. When the USB-driver for the HW is installed on the PC, this provides access to the linux console (TeraTerm 11520 8N1)

2 -> USB-A port: Can be used to connect USB-stick (FAT) to the HW. When the Linux image is booted the USB driver of the Linux System is recognizing the plugged in USB-stick.

Use

mount /dev/sda1 /mnt

to mount the USB stick to the file system. With

cd /mnt

one should be able to go into the USB stick folder/content. From there one is also able to execute built linux applications.

3 -> Ethernet port

4 -> external power supply input connector (5V)

5 -> Reset button (hidden under display shield)

6 -> 240x320 pixel touch display (/dev/fbo ad /dev/input/event0)

# How to build the Altia HMI

The ByteSizedDemo project was assigned to the "miniGL SW Render (MASTER) v13.2.1" template (NOT for customers!!!).

## Code Gen Options

Important: The FreeType runtime font engine has to be disabled, because first results lead to the HMI crashing (Segmentation Fault) when executed on HW.

Target Toolchain -> gcc_armeabi

Target CPU -> arm926ej-s

Target FPU -> (leave empty) -> requires an adaptation of the altmake_gcc_armeabi.mk make file (since this device does not have an FPU):

CDEFINES += \

MINIGL_INLINE=inline \

FS_INLINE=inline

CFLAGS += \

-mcpu=$(strip $(MINIGL_SRT_CPU)) \

-mthumb

ifeq ($(strip $(MINIGL_SRT_FPU)),)

CFLAGS +=

else

CFLAGS += -mfloat-abi=hard -mfpu=$(strip $(MINIGL_SRT_FPU))

endif  # ifeq ($(strip $(MINIGL_SRT_FPU)),)

CXXFLAGS += \

-mcpu=$(strip $(MINIGL_SRT_CPU)) \

-mthumb \

-fno-exceptions

ifeq ($(strip $(MINIGL_SRT_FPU)),)

CXXFLAGS +=

else

CXXFLAGS += -mfloat-abi=hard -mfpu=$(strip $(MINIGL_SRT_FPU))

endif  # ifeq ($(strip $(MINIGL_SRT_FPU)),)

Target Options -> -march=armv5te -mfloat-abi=soft -fno-short-enums

To build the libaltia.a from the generated code Windows version of gcc-arm-none-eabi-6.2.1 compiler was installed and 'set TOOLCHAIN_BASE_PATH= C:\gcc-arm-none-eabi-6.2.1" was set on CMD. Then run altmake.bat.

## Build the Executable

When the Altia HMI library is built (one should have a complete 'out' folder holding the built library, the BAM reflash folder and the header files). One can use a bytesizeddemo.c file together with a Makefile to build the bytesizeddemo executable in the Nuvoton Buildroot VMWare environment (https://www.nuvoton.com/resource-download.jsp?tp_GUID=SW1320200406183205):

Makefile:

.SUFFIXES : .x .o .c .s

HMI_DIR := .

HMI := ByteSizedDemo

CC := arm-linux-gcc

STRIP := arm-linux-strip

OBJCOPY := arm-linux-objcopy

INCLUDE := -I./out

LIBDIRS := -L./out

LIBS := -laltia

OBJFILES := $(HMI_DIR)/out/reflash/$(HMI)/altia_table_bin.o $(HMI_DIR)/out/reflash/$(HMI)/images/altia_images_bin.o $(HMI_DIR)/out/reflash/$(HMI)/fonts/altia_fonts_bin.o

TARGET = bytesizeddemo

SRCS := bytesizeddemo.c

prebuild:

$(OBJCOPY) -I binary -O elf32-littlearm -B arm --rename-section .data=.rodata,alloc,load,readonly,data,contents $(HMI_DIR)/out/reflash/$(HMI)/table.bin $(HMI_DIR)/out/reflash/$(HMI)/altia_table_bin.o;

$(OBJCOPY) -I binary -O elf32-littlearm -B arm --rename-section .data=.rodata,alloc,load,readonly,data,contents $(HMI_DIR)/out/reflash/$(HMI)/images/altiaImageDataPartition0.bin $(HMI_DIR)/out/reflash/$(HMI)/images/altia_images_bin.o;

$(OBJCOPY) -I binary -O elf32-littlearm -B arm --rename-section .data=.rodata,alloc,load,readonly,data,contents $(HMI_DIR)/out/reflash/$(HMI)/fonts/altiaImageDataPartition0.bin $(HMI_DIR)/out/reflash/$(HMI)/fonts/altia_fonts_bin.o;

all:

$(CC) -Wl,-Map -Wl,bytesizeddemo.map -mthumb -static -DMINIGL_UNICODE $(INCLUDE) $(SRCS) $(LIBDIRS) $(LIBS) $(OBJFILES) -o $(TARGET)

#        $(STRIP) $(TARGET)

clean:

rm -f *.o

rm -f *.x

rm -f *.flat

rm -f *.map

rm -f temp

rm -f *.img

rm -f $(TARGET)

rm -f *.gdb

rm -f *.bak

bytesizeddemo.c

#include <stdio.h>

#include <stdlib.h>

#include <fcntl.h>

#include <linux/fb.h>

#include <sys/mman.h>

#include <string.h>

#include <linux/input.h>

#include <sys/resource.h>

#include "miniGL.h"

#include "miniGL_software_render.h"

#include "miniGLFunctions.h"

//#define DEBUG

// handle touch

const char *ev_type[EV_CNT] = {

[EV_SYN]       = "EV_SYN",

[EV_KEY]       = "EV_KEY",

[EV_REL]       = "EV_REL",

[EV_ABS]       = "EV_ABS",

[EV_MSC]       = "EV_MSC",

[EV_SW]        = "EV_SW",

[EV_LED]       = "EV_LED",

[EV_SND]       = "EV_SND",

[EV_REP]       = "EV_REP",

[EV_FF]        = "EV_FF",

[EV_PWR]       = "EV_PWR",

[EV_FF_STATUS] = "EV_FF_STATUS",

[EV_MAX]       = "EV_MAX",

};

const char *ev_code_syn[SYN_CNT] = {

[SYN_REPORT]    = "SYN_REPORT",

[SYN_CONFIG]    = "SYN_CONFIG",

[SYN_MT_REPORT] = "SYN_MT_REPORT",

[SYN_DROPPED]   = "SYN_DROPPED",

[SYN_MAX]       = "SYN_MAX",

};

const char *ev_code_abs[ABS_CNT] = {

[ABS_X]    = "ABS_X",

[ABS_Y]    = "ABS_Y",

[ABS_PRESSURE] = "ABS_PRESSURE",

[ABS_MAX] = "ABS_MAX",

};

const char *ev_code_rel[REL_CNT] = {

[REL_X]      = "REL_X",

[REL_Y]      = "REL_Y",

[REL_Z]      = "REL_Z",

[REL_RX]     = "REL_RX",

[REL_RY]     = "REL_RY",

[REL_RZ]     = "REL_RZ",

[REL_HWHEEL] = "REL_WHEEL",

[REL_DIAL]   = "REL_DIAL",

[REL_WHEEL]  = "REL_WHEEL",

[REL_MISC]   = "REL_MISC",

[REL_MAX]    = "REL_MAX",

};

const char *ev_code_key[KEY_CNT] = {

[BTN_LEFT]    = "BTN_LEFT",

[BTN_RIGHT]   = "BTN_RIGHT",

[BTN_MIDDLE]  = "BTN_MIDDLE",

[BTN_SIDE]    = "BTN_SIDE",

[BTN_EXTRA]   = "BTN_EXTRA",

[BTN_FORWARD] = "BTN_FORWARD",

[BTN_BACK]    = "BTN_BACK",

[BTN_TASK]    = "BTN_TASK",

[BTN_TOUCH]   = "BTN_TOUCH",

[KEY_MAX]     = "KEY_MAX",

};

struct input_event ie;

void print_event(struct input_event *ie)

{

switch (ie->type) {

case EV_SYN:

printf("time:%ld.%06ld\ttype:%s\tcode:%s\tvalue:%d\n",

ie->time.tv_sec, ie->time.tv_usec, ev_type[ie->type],

ev_code_syn[ie->code], ie->value);

break;

case EV_ABS:

printf("time:%ld.%06ld\ttype:%s\tcode:%s\tvalue:%d\n",

ie->time.tv_sec, ie->time.tv_usec, ev_type[ie->type],

ev_code_abs[ie->code], ie->value);

break;

case EV_REL:

printf("time:%ld.%06ld\ttype:%s\tcode:%s\tvalue:%d\n",

ie->time.tv_sec, ie->time.tv_usec, ev_type[ie->type],

ev_code_rel[ie->code], ie->value);

break;

case EV_KEY:

printf("time:%ld.%06ld\ttype:%s\tcode:%s\tvalue:%d\n",

ie->time.tv_sec, ie->time.tv_usec, ev_type[ie->type],

ev_code_key[ie->code], ie->value);

break;

default:

break;

}

}

int touch_fd;

#define TOUCH_DEV "/dev/input/event0"

#define FBDEV "/dev/fb0"

int fbfd;

struct fb_var_screeninfo vinfo;

unsigned short int *displayFramebuffer = NULL;

/* handle FBDEV */

unsigned short int *init_fbdev()

{

unsigned short int *fbpointer = NULL;

int fd = open(FBDEV, O_RDWR);

if(fd >= 0)

{

int fbsize;

ioctl(fd, FBIOGET_VSCREENINFO, &vinfo);

fbsize = vinfo.xres * vinfo.yres * (vinfo.bits_per_pixel/8);

fbpointer = (unsigned short int *)mmap(0, fbsize, PROT_READ | PROT_WRITE, MAP_SHARED, fd, (off_t)0);

}

return fbpointer;

}

void fillFrameBuffer(unsigned short int *fbbuffer, short int color)

{

int x,y;

for(y = 0; y < vinfo.yres; y++)

{

for(x = 0; x < vinfo.xres; x++)

{

*(fbbuffer + y * vinfo.xres + x) = color;

}

}

}

void copyFrameBuffer(unsigned short int *source, unsigned short int *destination)

{

#if 1

int x,y;

for(y = 0; y < vinfo.yres; y++)

{

for(x = 0; x < vinfo.xres; x++)

{

*(destination + y * vinfo.xres + x) = *(source + y * vinfo.xres + x);

}

}

#endif

}

/* framebuffer */

#define DISPLAY_WIDTH 240

#define DISPLAY_HEIGHT 320

#define BYTES_PER_PIXEL 2

unsigned char *framebuffer = NULL;

extern const unsigned int  _binary___out_reflash_ByteSizedDemo_table_bin_start;

MINIGL_UINT8 *altiaTable = (MINIGL_UINT8 *)&_binary___out_reflash_ByteSizedDemo_table_bin_start;

extern const unsigned int  _binary___out_reflash_ByteSizedDemo_images_altiaImageDataPartition0_bin_start;

MINIGL_UINT8 *altiaImages = (MINIGL_UINT8 *)&_binary___out_reflash_ByteSizedDemo_images_altiaImageDataPartition0_bin_start;

extern const unsigned int  _binary___out_reflash_ByteSizedDemo_fonts_altiaImageDataPartition0_bin_start;

MINIGL_UINT8 *altiaFonts = (MINIGL_UINT8 *)&_binary___out_reflash_ByteSizedDemo_fonts_altiaImageDataPartition0_bin_start;

MINIGL_UINT8 * bsp_ReflashQueryString(MINIGL_CONST MINIGL_UINT16 * string)

{

MINIGL_CONST MINIGL_UINT8 *address = NULL;

char local_string[128] = {0};

MINIGL_UINT32 count = 0;

if(string == NULL)

{

return altiaTable;

}

while((count < 128) && (string[count] != 0x0000))

{

local_string[count] = (MINIGL_UINT8)string[count];

count++;

}

if(0 == strcmp(local_string, "\\images"))

{

address = altiaImages;

}

#if 1

else if(0 == strcmp(local_string, "\\fonts"))

{

address = altiaFonts;

}

#endif

return (MINIGL_UINT8 *)address;

}

void* bsp_GetBackFrameBuffer()

{

#ifdef DEBUG

printf("bsp_GetBackFrameBuffer() called!\n");

#endif

return (void *)framebuffer;

}

void updateRectangle(unsigned char *source, unsigned char *destination, int x0, int y0, int x1, int y1)

{

int line_cnt;

int lines = y1 - y0;

int cpy_length = x1 - x0;

unsigned char *src = source + y0 * vinfo.xres + x0;

unsigned char *dst = destination + y0 * vinfo.xres + x0;

for(line_cnt = 0; line_cnt < lines; line_cnt++)

{

memcpy((void *)dst, (void *)src, (size_t)(cpy_length * vinfo.bits_per_pixel / 8));

src += vinfo.xres * vinfo.bits_per_pixel / 8;

dst += vinfo.xres * vinfo.bits_per_pixel / 8;

}

}

MINIGL_INT bsp_DriverFlushed(MINIGL_CONST MINIGL_INT16 dirtyExtentCount, MINIGL_CONST BSP_CLIPPING_RECTANGLE * dirtyExtentList)

{

int i;

#ifdef DEBUG

printf("bsp_DriverFlushed() called!\n");

printf("Source Buffer Address: 0x%x\n", (unsigned int)framebuffer);

printf("Destination Buffer Address: 0x%x\n", (unsigned int)framebuffer);

printf("dirtyExtentCount: %i\n", dirtyExtentCount);

#endif /* DEBUG */

for(i = 0; i < dirtyExtentCount; i++)

{

#ifdef DEBUG

printf("dirtyExtentList[%i].x0 = %i\n", i, dirtyExtentList[i].x0);

printf("dirtyExtentList[%i].y0 = %i\n", i, dirtyExtentList[i].y0);

printf("dirtyExtentList[%i].x1 = %i\n", i, dirtyExtentList[i].x1);

printf("dirtyExtentList[%i].y1 = %i\n", i, dirtyExtentList[i].y1);

#endif

updateRectangle((unsigned char *)framebuffer, (unsigned char *)displayFramebuffer, dirtyExtentList[i].x0, dirtyExtentList[i].y0, dirtyExtentList[i].x1, dirtyExtentList[i].y1);

}

//    copyFrameBuffer((unsigned short int *)framebuffer, (unsigned short int *)displayFramebuffer);

}

void bsp_ErrorHandler(MINIGL_CONST MINIGL_UINT32 errorCode, MINIGL_CONST MINIGL_UINT32 majorCode, MINIGL_CONST MINIGL_UINT32 minorCode)

{

printf("bsp_ErrorHandler() called!\n");

printf("Error Code: %lu\r\n", errorCode);

printf("Error MajorCode: %lu\r\n", majorCode);

printf("Error MinorCode: %lu\r\n", minorCode);

}

typedef struct

{

/*

* Set once during initialization

*/

MINIGL_UINT16 dispWidth, dispHeight;       /* dimensions of associated display */

/*

* Event based state updated by bsp_GetEvent()

*/

MINIGL_UINT16 evFlag;                         /* OR of EVENT_* bits */

MINIGL_UINT32 pen;                       /* pending mouse or touch */

MINIGL_UINT32 x;

MINIGL_UINT32 y;

} INPUT_DEVICE;

INPUT_DEVICE input_device;

/*

* Bit masks to use for evFlag field in INPUT_DEVICE struct

*/

#define EVENT_ABS_POS                           0x0001

#define EVENT_BUTTON_PRESSED                    0x0002

#define EVENT_BUTTON_RELEASED                   0x0004

#define EVENT_SYN_DROPPED                       0x0008

#define EVENT_SYN_MT_REPORT                     0x0010

#define EVENT_ABS_MT_ANY                        0x0020

#define EVENT_ABS_MT_POS                        0x0040

#define EVENT_ABS_MT_SLOT                       0x0080

#define EVENT_ABS_MT_PRESSURE                   0x0100

/*

* Macros to set/clear/test evFlag bit fields in INPUT_DEVICE struct

*/

#define EVENT_FLAG_SET(d,b)                     ((d)->evFlag |= (b))

#define EVENT_FLAG_CLR(d,b)                     ((d)->evFlag &= ~(b))

#define EVENT_FLAG_IS_SET(d,b)                  (0 != ((d)->evFlag & (b)))

#define EVENT_FLAG_IS_CLR(d,b)                  (0 == ((d)->evFlag & (b)))

//#define DEBUG_TOUCH

MINIGL_INT bsp_GetEvent(BSP_STIMULUS_EVENT * event, MINIGL_INT32 timeout)

{

int read_bytes = 0;

int ret_value = 0;

#ifdef DEBUG_TOUCH

printf("bsp_GetEvent() called!\n");

#endif  /* DEBUG_TOUCH */

do

{

read_bytes = read(touch_fd, &ie, sizeof(struct input_event));

if(read_bytes >= 0)

{

#ifdef DEBUG_TOUCH

print_event(&ie);

#endif /* DEBUG_TOUCH */

if((ie.type == EV_SYN) && (ie.code == SYN_REPORT))

{

if(EVENT_FLAG_IS_CLR(&input_device, EVENT_SYN_DROPPED))

{

if(EVENT_FLAG_IS_SET(&input_device, EVENT_ABS_POS))

{

EVENT_FLAG_CLR(&input_device, EVENT_ABS_POS);

event->x = input_device.x;

event->y = input_device.y;

ret_value = 1;

}

if(EVENT_FLAG_IS_SET(&input_device, EVENT_BUTTON_PRESSED))

{

EVENT_FLAG_CLR(&input_device, EVENT_BUTTON_PRESSED);

event->pen = 1;

ret_value = 1;

}

if(EVENT_FLAG_IS_SET(&input_device, EVENT_BUTTON_RELEASED))

{

EVENT_FLAG_CLR(&input_device, EVENT_BUTTON_RELEASED);

event->pen = 0;

ret_value = 1;

}

// clear all event flags

input_device.evFlag = 0;

}

}

else if((ie.type == EV_SYN) && (ie.code == SYN_DROPPED))

{

EVENT_FLAG_SET(&input_device, EVENT_SYN_DROPPED);

}

else if((ie.type == EV_ABS) && (ie.code == ABS_X))

{

input_device.x = (MINIGL_INT16)((240 / 4096.9) * ie.value);

EVENT_FLAG_SET(&input_device, EVENT_ABS_POS);

}

else if((ie.type == EV_ABS) && (ie.code == ABS_Y))

{

input_device.y = (MINIGL_INT16)((320 / 4096.9) * ie.value);

EVENT_FLAG_SET(&input_device, EVENT_ABS_POS);

}

else if((ie.type == EV_KEY) && (ie.code == BTN_TOUCH))

{

input_device.pen = ie.value;

if(ie.value == 1) EVENT_FLAG_SET(&input_device, EVENT_BUTTON_PRESSED);

if(ie.value == 0) EVENT_FLAG_SET(&input_device, EVENT_BUTTON_RELEASED);

}

}

} while( read_bytes > 0 );

return ret_value;

}

int main(void)

{

const rlim_t kStackSize = 64L * 1024L;   // min stack size = 64 kb

struct rlimit rl;

int result;

result = getrlimit(RLIMIT_STACK, &rl);

if (result == 0)

{

if (rl.rlim_cur < kStackSize)

{

rl.rlim_cur = kStackSize;

result = setrlimit(RLIMIT_STACK, &rl);

if (result != 0)

{

fprintf(stderr, "setrlimit returned result = %d\n", result);

}

}

}

// open touch device

if ((touch_fd = open(TOUCH_DEV, O_RDONLY | O_NONBLOCK)) == -1) {

printf("Failed to open touch device!");

while(1);

}

// open display device

displayFramebuffer = init_fbdev();

framebuffer = (unsigned char *)malloc((size_t)(DISPLAY_WIDTH * DISPLAY_HEIGHT * BYTES_PER_PIXEL));

// start Altia HMI

if(altiaInitDriver() < 0)

{

printf("altiaInitDriver() failed!\n");

while(1);

}

/* draw first screen */

altiaSendEvent(ALT_ANIM(TimerEnable), 0);

altiaFlushOutput();

for(;;)

{

altiaPending();

/* run altia HMI */

altiaSendEvent(ALT_ANIM(do_timer), 1);

altiaFlushOutput();

}

return 0;

}

To build the demo run the VMWare Linux PC:

> make prebuild

> make all

# How to Deploy and Run ByteSizedDemo

The ready built demo executable can be downloaded from here: https://altia.box.com/s/0hxfxrrqhh8u9q2fs0fgyyckxxmmlxv3

Copy the file bytesizeddemo onto an USB-stick and connect the USB stick to USB-port (2), mount the USB-stick to the Linux file system: mount /dev/sda1 /mnt

Change tothe folder 'mnt': cd /mnt

Run the demo: ./bytesizeddemo

Remark: The HW comes with an emWIN demo auto-started at boot. To run the Altia demo login to the Linux console (root, No Password) and execute: killall GUIDemo

This will stop the GUIDemo and the Altia ByteSizedDemo can be executed.